FastLink PCI Ethernet Adapter

DC21X4 miniport driver for Windows 95 [tm]

Version v4.22

25-Jun-1997

1. Copyright Notice:

Windows, Windows NT and Windows 95 are trademarks of

Microsoft Corporation.

Microsoft, MS, MS-DOS are registered trademarks of

Microsoft Corporation.

All other trademarks and registered trademarks are the

property of their respective holders.

2. Overview:

This file explains how to install the Windows 95 driver

The driver is currently supported for motherboards and option

boards based on the Digital Semiconductor 2104x/2114x 10/100 mbps

Ethernet Controller.

3. Requirements:

A.  Hardware Requirements:

80386 or higher microprocessor based computer

with an EISA or PCI bus.

Digital Semiconductor 2104x/2114x 10/100 mbps Ethernet Controller

B.  Software Revision Requirements:

Windows 95 Final Beta Release or later

4. Files and directories:

---------------------------------------------------------------------

INSTALL.DOC         This file.

---------------------------------------------------

DC21X4.SYS          The miniport driver.

---------------------------------------------------

LTPCI.INF           The driver's setup file

---------------------------------------------------------------------

5. Installing the Driver on Windows 95:

5.1 Start Windows 95 or OSR2 without pluging in FastLink adapter

5.2 Run INSTALL.EXE from driver diskette, and install Windows 95 drivers to

a directory, for example C:\FASTLINK, on the hard disk.

5.3 If you have installed Digital Equipment's PCI Fast Ethernet 21140 based

Adapter, please remove it by following the following steps:

5.3.1 open System icon from Control Panel

5.3.2 select Network Adapter, and select PCI Fast Ethernet DECchip 21140

based Adapter, and click Remove button to remove it

5.4 Open Network icon from Control Panel

5.5 Click Add button, double-click Adapter, and select Have Disk

5.6 When you are asked to enter path name for an INF file, enter the path

where you installed FastLink drivers in step 5.2 previously, for

example, C:\FASTLINK.

5.7 Select Digital Semiconductor 21140 based 10/100 mpbs Ethernet Controller

and click OK

5.8 In the Network dialog box, you can click Add again to add additional

Protocols or Client Services, or click File and Print Sharing to let

others share you files and printers. After all are added, click OK

5.9 The system will ask you to insert Windows 95 CD-ROM in order to copy

needed files.

5.10 Restart the computer

5.11 Windows 95 will auto-dectect the PCI Ethernet Controller at startup,

select Driver From Disk Provided by Hardware Manufacturer, and click

OK.

5.12 If you are asked to insert the manufacture's installation disk...,

then enter the path name on which you installed FastLink drivers in

in step 5.2 previously, for example, C:\FASTLINK.

5.13 Follow instructions on screen to finish setup

6. Parameters

To overwrite the initial values set by the EISA Configuration

Utility (ECU) or overwrite the driver's default values,

add or edit the following values in the Registry

Parameters subtree:

ConnectionType:

------------------------------------------------------------------------

Digital Semiconductor 2114x based 10/100 mbps Ethernet Controller

-------------------------------------------------------------------------

o 100BaseTx (TP 100)          To connect 100BaseTx Ethernet hubs

o 100BaseTx (TP 100) FDx      To connect 100BaseTx Full Duplex hubs

o 100BaseFx                   To connect 100BaseFx Ethernet hubs

o 100BaseFx Full_Duplex       To connect 100BaseFx Full Duplex hubs

CacheLineSize = { 16 | 32 | 64 | 128 }

16 bytes

32 bytes (default)

64 bytes

128 bytes

InterruptMitigation = {Enabled | Disabled} (21140,21143/2)

Enable/disable the Interrupt Mitigation

algorithm.

InterruptThreshold = {number_of_interrupt/second} (default: 500)

Maximum number of interrupt/second allowed before

the Receive and Transmit interrupts get masked.

Meaningfull only when InterruptMitigation is enabled.

FrameThreshold = {number_of_frame/second} (default: 400)

Minimum number of processed frame/second allowed

before the Receive and Transmit interrupts get

unmasked.

Meaningfull only when InterruptMitigation is enabled.

MapRegisters =  {4,8,16,32,64}

Number of map registers allocated

by the adapter.

MemoryReadMultiple = { Disabled | Enabled }  (21142,21143 only)

Enable/disable PCI Memory_Read_Multiple command.

MRM should be enabled when the DC21X4 is connected

to the TI1130 PCI/CardBus bridge.

NwayProtocol = { Enabled | Disabled }  (21041 and

21142 Rev1.0,1.1 only)

Enable/disable the 10BT/10BT_Full Duplex Autonegotiation

medium in Autosense mode.

ReceiveBuffers = {8..64}

Number of buffers allocated by the

driver in the Receive ring.

SoftwareCRC = { 0 |  1 }  (21040 rev 1.x, 2.0 & 2.2 only)

0 The CRC is generated by the adapter

1 (default) The CRC is generated by software for

transmit packets bigger than the transmit threshold.

NOTE: This option affects performances and could be

disabled if the adapter does not report transmit

underruns in its statistic counters

UnderrunRetry =  {2,4,6,8}

Maximun number of retransmission attempts of

an underrun packet.

UnderrunThreshold = {10,20,50,100} (21140,21143/2 only)

Maximun number of underrun occurences before the

controller switches to the next threshold value.

NoCableIndication =  {Enabled | Disabled}

Check the LinkStatus before sending a packet and

retrieves NDIS_STATUS_NO_CABLE if there is not Link

indication. When it is disabled, it will retrieve the

NDIS_STATUS_SUCCESS message (workaround for the Win95

wrapper handling problem of the NO_CABLE status message).

The following values allow to overwrite the DC21X4 default setting.

Refer to the Digital Semiconductor 2104x/2114x 10/100 mbps Ethernet Controller

Engineering specifications for detailed description:

BackPressure = { 0 | 1 }  (not 21040 rev1)

CSR6<BP>    0  Disabled (default)

1  Enabled

BurstLength =  { 0 | 1 | 2 | 4 | 8 | 16 | 32 }

CSR0<PBL>   0  unlimited     (default) - (not 21040, 21041 & 21140 rev1)

1  doubleword

2  doublewords

4  doublewords

8  doublewords

16  doublewords   (default for 21040, 21041 & 21140 rev1)

32  doublewords   (not 21040 & 21041)

CaptureEffect = { 0 | 1 } (not 21040 rev1)

CSR6<CA>    0  Disabled (Default)

1  Enabled

FifoArbitration = { 0 | 1 }

CSR0<BAR>   0 Rcv/Txm Arbitration (default)

1 Round_Robin

SnoozeMode =    {Enabled|Disabled}

CFDA<SM>    0 Disabled

1 Enabled

StopBackoffCounter = { 0 | 1 }  (not 21040 rev1)

CSR6<SB>    0  Disabled (Default)

1  Enabled

StoreAndForward =  {Enabled|Disabled} (21140, 21143/2 only)

Select the StoreAndForward mode if high

PCI latencies causes 100Mbps Transmit Underruns

CSR6<SF>    1 Enabled

0 Disabled

TransmitThreshold = {value}

Transmit Threshold values for 10Mbps media.

CSR6<TTM,TR>  72 bytes

96 bytes

128 bytes

160 bytes

TransmitThreshold100 =  {value} (21140, 21143/2 only)

Transmit Threshold values for 100Mbps media.

Meaningless when StoreAndForward is set

CSR6<TTM,TR> 128 bytes

256 bytes

512 bytes

1024 bytes

7. Driver Revision number

To retrieve the driver revision number, query the Registry:

HKEY_LOCAL_MACHINE\

System\CurrentControlSet\Services\Class\Net\"num"\NDIS

MajorDriverVersion

MinorDriverVersion

8. Error messages

DC21X4_ERRMSG_REGISTRY              0x01

DC21X4_ERRMSG_ALLOC_MEMORY          0x02

DC21X4_ERRMSG_SROM                  0x03

DC21X4_ERRMSG_MEDIA                 0x04

DC21X4_ERRMSG_LOAD_CAM              0x05

DC21X4_ERRMSG_SYSTEM_ERROR          0x06

DC21X4_ERRMSG_TXM_JABBER_TIMEOUT    0x07

8.1 Driver initialization errors:

o NDIS_ERROR_CODE_UNSUPPORTED_CONFIGURATION

DC21X4_ERRMSG_REGISTRY

The AdapterType Registry's key is missing or its value is

unsupported by the adapter board

o NDIS_ERROR_CODE_ADAPTER_NOT_FOUND

No board matching the AdapterCFID Registry's key value was found

plugged into the PCI bus or in the slot specified in the Registry

o NDIS_ERROR_CODE_OUT_OF_RESOURCES

DC21X4_ERRMSG_ALLOC_MEMORY

Not enough memory to allocate space for the Adapter data block

o NDIS_ERROR_CODE_INVALID_VALUE_FROM_ADAPTER

DC21X4_ERRMSG_SROM

Invalid data read from the adapter's serial ROM

o NDIS_ERROR_CODE_NETWORK_ADDRESS

No readable burnt_in or software configured station address

o NDIS_ERROR_CODE_UNSUPPORTED_CONFIGURATION

DC21X4_ERRMSG_MEDIA

The selected media port is not supported by the board.

o NDIS_ERROR_CODE_INTERRUPT_CONNECT

Interrupt initialization failed

o NDIS_ERROR_CODE_TIMEOUT

DC21X4_ERRMSG_LOAD_CAM

DC21X4 CAM initialization failed

8.2 Driver run time errors:

o NDIS_ERROR_CODE_HARDWARE_FAILURE

DC21X4_ERRMSG_SYSTEM_ERROR

System_Error interrupt (bus parity error)

o NDIS_ERROR_CODE_HARDWARE_FAILURE

DC21X4_ERRMSG_TXM_JABBER_TIMEOUT

Transmit Jabber timer expired