FastLink PCI Ethernet Adapter

DC21X4 NDIS 4.0 miniport driver for Intel Windows NT [tm] 4.0

Version v4.22

25-Jun-1997

1. Copyright Notice:

Windows, Windows NT, and Windows95 are trademarks of

Microsoft Corporation.

Microsoft, MS, MS-DOS are registered trademarks of

Microsoft Corporation.

All other trademarks and registered trademarks are the

property of their respective holders.

2. Overview:

The driver is currently supported for motherboards and option

boards based on the Digital Semiconductor 2104x/2114x 10/100 mbps

Ethernet Controller.

3. Requirements:

A.  Hardware Requirements:

Digital Semiconductor 2104x/2114x 10/100 mbps Ethernet Controller

B.  Software Revision Requirements:

Windows NT Version 4.0 or later

4. Files and directories:

---------------------------------------------------------------------

INSTALL.DOC         This file.

---------------------------------------------------

OEMSETUP.INF        The driver's setup file for Windows NT

---------------------------------------------------

DC21X4.SYS          The driver's executable

---------------------------------------------------------------------

5. Installing the DC21X4 Driver on Windows NT:

1. Run INSTALL.EXE located in driver diskette, and install Windows

NT drivers to a directory, e.g. C:\FASTLINK, on hard disk.

2. Click "Start", select "Settings", and click the Control Panel.

3. From the Control Panel menu select the Network icon.

4. In the Network Configuration dialog box, select "Add.." button.

The "Select Network Component Type" dialog box will appear.

Select "Adapter".

5. Select "Have Disk".

6. You will be asked to enter the path name of an INF file,

enter the directory name where you installed drivers, for

example, C:\FASTLINK.

7. Select "Digital Semiconductor 21140 based 10/100 mpbs Ethernet

Controller".

8. Windows NT will copy needed files. You need to insert Windows

NT CD-ROM or disks to complete setup.

9. If needed, modify the values in the registry (see section 6 below).

10. Shutdown and reboot the system.

6. Parameters

To overwrite the initial values set by the EISA Configuration

Utility (ECU) or overwrite the driver's default values,

add or edit the following values in the Registry

DC21X4<adapter_number> Parameters subtree:

CacheLineSize = REG_DWORD { 16 | 32 | 64 | 128 }

16 bytes

32 bytes (default)

64 bytes

128 bytes

ExtraReceiveBuffers = REG_DWORD {value} (default: 16)

Number of buffers allocated by the

driver in the Receive look_aside list.

ExtraReceivePackets = REG_DWORD {value} (default: 100)

Number of packets preallocated by the

driver.

InterruptMitigation = REG_DWORD  {0 | 1} (21140,21143/2)

0  Disabled (default)

1  Enabled

Enable/disable the Interrupt Mitigation

algorithm.

InterruptThreshold = REG_DWORD {value} (default: 500)

Maximum number of interrupt/second allowed before

the Receive and Transmit interrupts get masked.

Meaningfull only when InterruptMitigation is enabled.

FrameThreshold = REG_DWORD {value} (default: 400)

Minimum number of processed frame/second allowed

before the Receive and Transmit interrupts get

unmasked.

Meaningfull only when InterruptMitigation is enabled.

MemoryReadMultiple = REG_DWORD { 0 | 1 }  (21142,2143 only)

0  Disabled (default)

1  Enabled

Enable/disable PCI Memory_Read_Multiple command.

MRM should be enabled when the DC21X4 is connected

to the TI1130 PCI/CardBus bridge.

NwayProtocol = REG_DWORD { 0 | 1 }  (21041 and

21142 Rev1.0,1.1 only)

0  Disabled

1  Enabled  (default)

Enable/disable the 10BT/10BT_FD Autonegotiation

medium in Autosense mode.

ReceiveBuffers = REG_DWORD {8..64} (default: 16)

Number of buffers allocated by the

driver in the Receive ring.

SoftwareCRC = REG_DWORD { 0 |  1 }  (DC21040 rev 1.x, 2.0 & 2.2 only)

0 The CRC is generated by the adapter

1 (default) The CRC is generated by software for

transmit packets bigger than the transmit threshold.

NOTE: This option affects performances and could be

disabled if the adapter does not report transmit

underruns in its statistic counters

UnderrunRetry = REG_DWORD  number_of_retries  (default=2)

Maximun number of retransmission attempts of

an underrun packet.

UnderrunThreshold = REG_DWORD  number_of_underruns (default=10)  (21140 and 21143/2 only)

Maximun number of underrun occurences before the

controller switches to the next threshold value.

The following values allow to overwrite the DC21X4 default setting.

Refer to the Digital Semiconductor 2104x/2114x 10/100 mbps Ethernet Controller

Engineering specifications for detailed description:

BackPressure = REG_DWORD { 0 | 1 }  (not DC21040 rev1)

CSR6<BP>    0  Disabled (default)

1  Enabled

BurstLength = REG_DWORD  { 0 | 1 | 2 | 4 | 8 | 16 | 32 }

CSR0<PBL>   0  unlimited     (default) - (not 21040, 21041 & 21140 rev1)

1  doubleword

2  doublewords

4  doublewords

8  doublewords

16  doublewords   (default for 21040, 21041 & 21140 rev1)

32  doublewords   (not 21040 & 21041)

CaptureEffect = REG_DWORD { 0 | 1 } (not 21040 rev1)

CSR6<CA>    0  Disabled (Default)

1  Enabled

FifoArbitration = REG_DWORD { 0 | 1 }

CSR0<BAR>   0 Rcv/Txm Arbitration (default)

1 Round_Robin

SnoozeMode = REG_DWORD { 0 | 1 }

CFDA<SM>    0 Disabled (default)

1 Enabled

StopBackoffCounter = REG_DWORD { 0 | 1 }  (not 21040 rev1)

CSR6<SB>    0  Disabled (Default)

1  Enabled

StoreAndForward = REG_DWORD { 0 | 1 }  (21140, 21143/2 only)

Select the StoreAndForward mode if high

PCI latencies causes 100Mbps Transmit Underruns

CSR6<SF>    1 Enabled

0 Disabled (default)

TransmitThreshold = REG_DWORD { 72 | 96  | 128 | 160 }

Transmit Threshold values for 10Mbps media.

CSR6<TTM,TR>  72 bytes

96 bytes (default)

128 bytes

160 bytes

TransmitThreshold100 = REG_DWORD { 128 | 256 | 512 | 1024 } (21140, 21143/2 only)

Transmit Threshold values for 100Mbps media.

Meaningless when StoreAndForward is set

CSR6<TTM,TR> 128 bytes

256 bytes

512 bytes (default)

1024 bytes

These Registry values can be edited interactively through the Registry

editor (winnt\system32\regedt32.exe) or by invoking the regini command

Edit an DC21X4.ini file as described below, then invoke the command:

regini DC21X4.ini

DC21X4.ini format:

\Registry\Machine

System

CurrentControlSet

Services

DC21x4{adapter_number}

Parameters

{valuename} = {type}{value}

7. Driver Revision number

To retrieve the driver revision number either

select the file DC21X4.SYS and open the File Properties applet

or

query the Registry:

HKEY_LOCAL_MACHINE\

Software\DigitalEquipmentCorporation\DC21X4\CurrentVersion

MajorVersion

MinorVersion

8. Error messages

The error messages are logged in the Entry Log and can be seen using the Entry

Log Viewer (in WNT's Administration Tools group).

Each error message has a standard Ndis error code and an driver's additional

code.The additional codes are:

DC21X4_ERRMSG_REGISTRY              0x01

DC21X4_ERRMSG_ALLOC_MEMORY          0x02

DC21X4_ERRMSG_SROM                  0x03

DC21X4_ERRMSG_MEDIA                 0x04

DC21X4_ERRMSG_LOAD_CAM              0x05

DC21X4_ERRMSG_SYSTEM_ERROR          0x06

DC21X4_ERRMSG_TXM_JABBER_TIMEOUT    0x07

8.1 Driver initialization errors:

o NDIS_ERROR_CODE_UNSUPPORTED_CONFIGURATION

DC21X4_ERRMSG_REGISTRY

The AdapterType Registry's key is missing or its value is

unsupported by the adapter board

o NDIS_ERROR_CODE_ADAPTER_NOT_FOUND

No board matching the AdapterCFID Registry's key value was found

plugged into the PCI bus or in the slot specified in the Registry

o NDIS_ERROR_CODE_OUT_OF_RESOURCES

DC21X4_ERRMSG_ALLOC_MEMORY

Not enough memory to allocate space for the Adapter data block

o NDIS_ERROR_CODE_INVALID_VALUE_FROM_ADAPTER

DC21X4_ERRMSG_SROM

Invalid data read from the adapter's serial ROM

o NDIS_ERROR_CODE_NETWORK_ADDRESS

No readable burnt_in or software configured station address

o NDIS_ERROR_CODE_UNSUPPORTED_CONFIGURATION

DC21X4_ERRMSG_MEDIA

The selected media port is not supported by the board.

o NDIS_ERROR_CODE_INTERRUPT_CONNECT

Interrupt initialization failed

o NDIS_ERROR_CODE_TIMEOUT

DC21X4_ERRMSG_LOAD_CAM

DC21X4 CAM initialization failed

8.2 Driver run time errors:

o NDIS_ERROR_CODE_HARDWARE_FAILURE

DC21X4_ERRMSG_SYSTEM_ERROR

System_Error interrupt (bus parity error)

o NDIS_ERROR_CODE_HARDWARE_FAILURE

DC21X4_ERRMSG_TXM_JABBER_TIMEOUT

Transmit Jabber timer expired