FastLink PCI Ethernet Adapter

DC21X4 fullmac driver for Intel [tm] Windows NT [tm] 3.5

Version v4.22

10-Jul-1997

1. Copyright Notice:

Intel is a trademark of Intel Corporation.

Windows and Windows NT are trademarks of

Microsoft Corporation.

Microsoft, MS, MS-DOS are registered trademarks of

Microsoft Corporation.

All other trademarks and registered trademarks are the property

of their respective holders.

2. Overview:

This file explains how to install the Windows NT driver

for x86-based platforms.

The driver is currently supported for motherboards and option

boards based on the Digital Semiconductor 2104x/2114x 10/100

mbps Ethernet Controller.

3. Requirements:

A.  Hardware Requirements:

80386 or higher microprocessor based computer

with an EISA or PCI bus.

Digital Semiconductor 2104x/2114x 10/100 mbps Ethernet Controller.

B.  Software Revision Requirements:

Windows NT Version 3.5 or later

4. Files and directories:

---------------------------------------------------------------------

INSTALL.DOC         This file.

---------------------------------------------------

OEMSETUP.INF        The driver's setup file for Windows NT

---------------------------------------------------

DC21X4.SYS          The driver's executable

---------------------------------------------------------------------

5. Installing the DC21X4 Driver on Windows NT:

1. Run INSTALL.EXE located in driver diskette, and install Windows

NT drivers to a directory on hard disk.

2. From the Windows NT Main Menu, select the Control Panel icon.

3. From the Control Panel menu select the Network icon.

If a dialog box asks you "Do you want to install NT

Network?" select YES and Continue. Then place

the Windows NT CDROM in the CD-ROM reader.

Select the drive that matches the configuration of your system

and complete the install.

4. You will see several messages. To add a network adapter, select Yes

and then Continue

5. For each additional message, select continue.

6. From the Add Network Adapter dialog box, scroll through the menu to

select Other.

7. When the Insert Diskettes menu prompts you for a disk, insert the

DC21X4 driver installation diskette for Windows NT into drive A.

Then select OK.

8. Select your option

[Digital Semiconductor 21140 based 10/100 mbps Ethernet Controller]

from the Select OEM Option box, then select OK.

9. Modify the default distribution path in the Windows NT

Setup box to point to the correct drive and path, then click

on [Continue]

10. For PCI options, a Dialog box pops up to select the:

ConnectionType:

------------------------------------------------------------------------

Digital Semiconductor's DC21140\DC21140A

-------------------------------------------------------------------------

o 100BaseTx                     To connect 100BaseTx Ethernet hubs

o 100BaseTx Full Duplex         To connect 100BaseTx Full Duplex hubs

o 100BaseFx                     To connect 100BaseFx Ethernet hubs

o 100BaseFx Full_Duplex         To connect 100BaseFx Full Duplex hubs

11. Upon completion, the selected adapter is added to the

Installed Adapter Cards list of the Network Setting box

(the number prefixing the adapter is the adapter number).

12. Click on [Close] to complete the Network Setting

initialization.

13. If needed, modify the values in the registry (see section 6 below).

14. Shutdown and reboot the system.

6. Parameters

To overwrite the initial values set by the EISA Configuration

Utility (ECU) overwrite the driver's default values,

add or edit the following values in the Registry

DC21x4_<adapter_number> Parameters subtree:

CacheLineSize = REG_DWORD { 16 | 32 | 64 | 128 }

16 bytes

32 bytes (default)

64 bytes

128 bytes

MapRegisters = REG_DWORD {4,8,16}   (default: 16(alpha) | 8)

Number of map registers allocated

by the adapter.

MemoryReadMultiple = REG_DWORD { 0 | 1 }  (21142,21143 only)

0  Disabled (default)

1  Enabled

Enable/disable PCI Memory_Read_Multiple command.

MRM should be enabled when the DC21X4 is connected

to the TI1130 PCI/CardBus bridge.

ReceiveBuffers = REG_DWORD {8..64} (default: 16)

Number of buffers allocated by the

driver in the Receive ring.

SoftwareCRC = REG_DWORD { 0 |  1 }  (DC21040 rev 1.x, 2.0 & 2.2 only)

0 The CRC is generated by the adapter

1 (default) The CRC is generated by software for

transmit packets bigger than the transmit threshold.

NOTE: This option affects performances and could be

disabled if the adapter does not report transmit

underruns in its statistic counters

UnderrunThreshold = REG_DWORD  number_of_underruns (default=10)  (21140 and 21143/2 only)

Maximun number of underrun occurences before the

controller switches to the next threshold value.

The following values allow to overwrite the DC21X4 default setting.

Digital Semiconductor 2104x/2114x 10/100 mbps Ethernet Controller

Engineering specifications for detailled description:

BackPressure = REG_DWORD { 0 | 1 }  (not 21040 rev1)

CSR6<BP>    0  Disabed (default)

1  Enabled

BurstLength = REG_DWORD  { 0 | 1 | 2 | 4 | 8 | 16 | 32 }

CSR0<PBL>   0  unlimited     (default) - (not 21040, 21041 & 21140 rev1)

1  doubleword

2  doublewords

4  doublewords

8  doublewords

16  doublewords   (default for 21040, 21041 & 21140 rev1)

32  doublewords   (not 21040 & 21041)

CaptureEffect = REG_DWORD { 0 | 1 }  (not 21040 rev1)

CSR6<CA>    0  Disabed (Default)

1  Enabled

FifoArbitration = REG_DWORD { 0 | 1 }

CSR0<BAR>   0 Rcv/Txm Arbitration (default)

1 Round_Robin

SnoozeMode = REG_DWORD { 0 | 1 }

CFDA<SM>    0 Disabled (default)

1 Enabled

StopBackoffCounter = REG_DWORD { 0 | 1 }  (not 21040 rev1)

CSR6<SB>    0  Disabed (Default)

1  Enabled

StoreAndForward = REG_DWORD { 0 | 1 }  (21140, 21143/2 only)

Select the StoreAndForward mode if high

PCI latencies causes 100Mbps Transmit Underruns

CSR6<SF>    1 Enabled

0 Disabled (default)

TransmitThreshold = REG_DWORD { 72 | 96  | 128 | 160 }

Transmit Threshold value for the 10Mbps media.

CSR6<TTM,TR>  72 bytes

96 bytes (default)

128 bytes

160 bytes

TransmitThreshold100 = REG_DWORD { 128 | 256 | 512 | 1024}

Transmit Threshold value for the 100Mbps media.

Meaningless when StoreAndForward is set

CSR6<TTM,TR>  128 bytes

256 bytes

512 bytes (default)

1024 bytes

These Registry values can be edited interactively through the Registry

editor (winnt\system32\regedt32.exe) or by invoking the regini command

Edit an DC21X4.ini file as described below, then invoke the command:

regini DC21X4.ini

DC21X4.ini format:

\Registry\Machine

System

CurrentControlSet

Services

dc21x4_{adapter_number}

Parameters

{valuename} = {type}{value}

7. Error messages

The error messages are logged in the Entry Log and can be seen using the Entry Log Viewer (in

WNT's Administration Tools group).

Each error message has a standard Ndis error code and an driver's additional code.

The additional codes are:

DC21X4_ERRMSG_REGISTRY            0x01

DC21X4_ERRMSG_ALLOC_MEMORY        0x02

DC21X4_ERRMSG_SROM                0x03

DC21X4_ERRMSG_MEDIA               0x04

DC21X4_ERRMSG_LOAD_CAM            0x05

DC21X4_ERRMSG_SYSTEM_ERROR        0x06

DC21X4_ERRMSG_TXM_JABBER_TIMEOUT  0x07

7.1 Driver initialization errors:

o NDIS_ERROR_CODE_UNSUPPORTED_CONFIGURATION

DC21X4_ERRMSG_REGISTRY

The AdapterType Registry's key is missing or its value is

unsupported by the adapter board

o NDIS_ERROR_CODE_ADAPTER_NOT_FOUND

No board matching the AdapterCFID Registry's key value was found

plugged into the PCI bus or in the slot specified in the Registry

o NDIS_ERROR_CODE_OUT_OF_RESOURCES

DC21X4_ERRMSG_ALLOC_MEMORY

Not enough memory to allocate space for the Adapter data block

o NDIS_ERROR_CODE_INVALID_VALUE_FROM_ADAPTER

DC21X4_ERRMSG_SROM

Invalid data read from the adapter's serial ROM

o NDIS_ERROR_CODE_NETWORK_ADDRESS

No readable burnt_in or software configured station address

o NDIS_ERROR_CODE_UNSUPPORTED_CONFIGURATION

DC21X4_ERRMSG_MEDIA

The selected media port is not supported by the board.

o NDIS_ERROR_CODE_INTERRUPT_CONNECT

Interrupt initialization failed

o NDIS_ERROR_CODE_TIMEOUT

DC21X4_ERRMSG_LOAD_CAM

DC21X4 CAM initialization failed

7.2 Driver run time errors:

o NDIS_ERROR_CODE_HARDWARE_FAILURE

DC21X4_ERRMSG_SYSTEM_ERROR

System_Error interrupt (bus parity error)

o NDIS_ERROR_CODE_HARDWARE_FAILURE

DC21X4_ERRMSG_TXM_JABBER_TIMEOUT

Transmit Jabber timer expired